UNIVERSITY OF ECONOMY IN BYDGOSZCZ

BUSANI NCUBE

MULTI-INTERFACE SMART HOME CONTROLLER

CHAPTER 1 | INTRODUCTION

1.1 Background

Home automation has been around for longer than most people think. In 1970, X10 became the first home automation platform, marking an important milestone. It allowed digital data to be transmitted through household electrical wiring using radio signals. This marked the beginning of homes becoming more modern and technology based. As technology continued to grow, the phrase “smart home” was first used in 1984 by the American Home Builders Association. By the late 20th century, technology was advancing rapidly. Home computers and early smart appliances became popular in the 1980s and 1990s, and more people started using them at home. During the late 1990s, there was a big increase in home network development, mainly because high-speed internet became widely available. As smartphones became more popular in the late 2000s, smart homes also grew in popularity since they could now be managed through mobile apps.

As Wi-Fi improved and became widely available, smart home devices became easier to use. With mobile networks such as 4G and 5G, users were able to operate their home devices remotely using mobile applications. IoT home systems connect different home devices, allowing them to work together and be controlled easily, making life more convenient, comfortable, safe, and energy efficient. This growth is reflected in market data - the global smart home market was worth USD 147.52 billion in 2025 and is expected to reach USD 848.47 billion by 2034, growing about 21.4% each year. In 2025, North America held the largest share of the smart home market at 31.7%, while Asia Pacific is projected to experience the fastest growth over the coming years.

Home Automation Systems (HASs), which are part of the smart grid and work with Internet of Things (IoT), are one of the best solutions for improving energy efficiency. In early stages of smart home development, the focus was on automating devices and making life easier for users. Over time, saving energy and reducing costs became more important. Research shows that smart home systems using predictive optimization can save energy costs by about 35.98% to 38.22%. In addition to saving money, smart homes reduce environmental impact. In a simulated smart home environment, the system outperformed conventional energy systems by increasing efficiency by up to 72.3%, lowering energy costs by up to 61%, and reducing CO₂ emissions by more than 61%.

The costs of smart home technology (SHT) have been widely discussed in academic research. Some studies show that cost plays an important role in whether people choose to adopt SHT, and lack of familiarity can make people worried about the price. People face several barriers when adopting smart homes, such as lack of trust, poor understanding of the technology, money concerns, worries about privacy and security, fear of technology, and negative social influence. Technology adoption depends on how people understand and perceive its benefits compared to the risks. Even though there are many advantages, some users still worry about losing control and facing security issues. Concerns around security, privacy, and trust continue to be among the most significant obstacles preventing wider smart home technology adoption

The Internet of Things (IoT) refers to a network of devices connected to the internet that can share data and be programmed to do tasks. It plays a central role in the development of modern smart homes. Smart home systems have recently grown in popularity due to IoT. These systems make life easier and more efficient by using devices such as thermostats and voice assistants to improve comfort and convenience. The number of connected IoT devices worldwide reached 18.5 billion in 2024, showing a 12% rise from 2023. It is expected that IoT devices will continue to grow by 14%, reaching 21.1 billion by the end of 2025 and by 2030, IoT devices are predicted to reach 39 billion, growing at a rate of 13.2% from 2025. Artificial Intelligence is expected to play a major role in this growth because it increases the demand for device data. This growth is especially clear in the voice assistant market, where over 320 million devices are installed in homes worldwide.

This project addresses these challenges by creating a low-cost smart home controller using a Raspberry Pi, with Home Assistant serving as an accessible and affordable alternative to high priced commercial systems.

1.2 Significance of the Study

IoT and smart home technologies are growing quickly around the world. The global Industrial IoT market reached USD 325.7 billion in 2025 and is projected to rise to USD 944.8 billion by 2034 with Europe holding the largest share of this market. Europe is one of the most important markets for IoT development because of its strong industry, government support, and increasing use of digital technology in many sectors. At the same time, the smart home market in Europe is expected to grow from USD 23.14 billion in 2025 to more than USD 32 billion by 2031, thanks to energy efficiency laws and rising demand from buyers.

In Poland, the IoT market is growing quickly. It is projected to go from €5.96 billion in 2025 to €9.39 billion in 2029, at a rate of 12.06% per year, driven by strong demand from manufacturing, energy, and transport industries. The use of IoT is growing in Poland as many industries use it to become more efficient and productive. The rising interest in smart homes was clear at the 2026 Smart Home and IoT Expo in Warsaw, which attracted 8,044 professionals from 31 countries—a 47% increase compared to the previous year.

Despite the rising popularity of smart home technology, cost remains a significant barrier to widespread use. Commercial smart home setups in Europe typically cost between EUR 3,500 and EUR 8,000, making them unaffordable for many households in Poland and other Eastern European countries. This project addresses that challenge by developing a low-cost smart home system using a Raspberry Pi and Home Assistant, offering households in Poland a more affordable alternative to high priced commercial systems.

1.3 Aim of the Project

The aim of this project is to design and build a low-cost smart home automation system using a Raspberry Pi 4 and Home Assistant. The system uses different sensors and actuators, including a DHT22 temperature and humidity sensor, a PIR motion sensor, a relay module, and an SG90 servo motor, to monitor and automate home functions like temperature, motion detection, device control, and door locking. The completed system can be accessed through a dashboard on any smartphone or browser, showing that a fully functional smart home setup can be created without the high costs associated with commercial systems.

1.4 Objectives

The following objectives were set to guide the development of this project:

- To install and set up Home Assistant OS (operating system) on a Raspberry Pi 4 as the main platform for the smart home system.

- To implement a DHT22 sensor for real-time monitoring of indoor temperature and humidity.

- To connect and configure a PIR sensor to trigger automated responses when motion is detected.

- To wire and control a relay module for switching electrical devices on and off through the Home Assistant dashboard.

- To implement an SG90 servo motor to act as a door lock, controlled automatically by the system.

- To evaluate the system through functional testing and user feedback.

1.5 Scope and Limitations

The focus of this project is to build a functional smart home system as a prototype for academic purposes. This setup includes temperature and humidity monitoring, motion sensing, device control, and a simulated door lock, all accessible through a single dashboard. The project does not aim to replace commercial smart home systems but rather to show that a functional alternative can be built at low cost.

This project has some limitations. The system depends entirely on a stable Wi-Fi connection — if the network is unavailable, remote access to the dashboard is lost. The system operates at prototype level and has not been tested for long-term reliability or use in a real home environment. Security features such as user authentication and encrypted communication are outside the scope of this project and are recommended for future development. Hardware development is limited to scheduled sessions at school, which restricts the total time available for physical testing and integration.

1.6 Structure of Thesis

This thesis is organised into six chapters. Chapter 1 introduces the project by providing background context, establishing the significance of the study, and defining the aim, objectives, scope, and limitations. Chapter 2 presents a review of existing literature covering smart home systems, the Home Assistant platform, Raspberry Pi in IoT applications, IIoT trends, sensor technologies, and a comparison of existing commercial systems. Chapter 3 describes the system design, including the architecture, hardware selection, automation logic, dashboard layout, and wiring diagrams. Chapter 4 describes how the system was implemented, including setting up all the hardware parts and creating the automations and dashboard. Chapter 5 explains how the system was tested, presents sensor accuracy results, evaluates automation performance, and shares feedback from users. Chapter 6 concludes the thesis by summarizing what was achieved, checking the objectives, noting limitations, suggesting future improvements, and reflecting on how the project relates to IIoT trends.

CHAPTER 2 | LITERATURE REVIEW

2.1 Overview of Smart Home Systems

The idea of a smart home has changed a lot over the past few decades, moving from simple wired control systems to advanced connected environments using Internet Of Things (IoT) and artificial intelligence. A smart home can be defined as a connected living space where appliances and systems can be controlled, monitored, and managed remotely through devices like smartphones, tablets, and voice-controlled interfaces. These systems combine technologies such as IoT, cloud computing, artificial intelligence, and embedded sensors to enable centralized management and remote control of household devices. These systems primarily provide better comfort, convenience, energy efficiency, and security, while also automating daily tasks that would otherwise need direct human involvement.

Smart home development began in the 1970s with the X10 platform, a system that used existing electrical wiring to transmit digital signals.The concept of smart homes grew further in the late 1990s with the rise of high-speed internet, and it accelerated again in the 2000s with the spread of smartphones and IoT devices. Research activity has reflected this development trend. According to Park and Han (2025), smart home publications increased sharply after 2014, with most academic work produced between 2018 and 2023, indicating growing technological maturity and broader applications.

The global market also demonstrates this continued growth. The smart home market was valued at USD 147.52 billion in 2025 and is projected to reach USD 848.47 billion by 2034, growing at a compound annual growth rate of 21.40% . North America currently holds the largest market share at 31.70%, while the Asia Pacific region is expected to grow the fastest due to urbanisation, rising incomes, and increased adoption of IoT devices. European adoption of smart home technologies is shaped by factors such as hedonic motivation (the enjoyment from using the technology), price value, and social influence, while energy management is becoming important due to increasing energy costs..

From an infrastructure perspective, Home Automation Systems rely on communication networks to collect, distribute, and process data within the home. Wired systems have several limitations, such as high installation costs, limited coverage range, and a lack of mobility. Wireless Sensor and Actuator Networks (WSANs) are a promising alternative to wired systems, offering better scalability, greater flexibility, and simpler integration with other networks. Research has shown that shifting from wired to wireless networks supports the use of computational intelligence to enhance decision-making in home automation.

Modern smart home systems use a layered architecture that consists of perception, network, edge, and cloud layers, along with an application layer that provides the user interface. The integration of AI and IoT has improved smart home systems by allowing devices to learn user habits and preferences, and automatically adjust to daily routines and environmental conditions without manual control. Ezugwu et al. (2025) notes that, smart home systems are likely to evolve into ambient intelligence environments that automatically respond to occupants’ needs. This shift will be driven by machine learning and deep learning, enabling more personalized and predictive automation.

2.2 Home Assistant as a Platform

Home Assistant is a free, open-source home automation platform developed and maintained by the Open Home Foundation, designed around the principles of local data control and user privacy. It has become one of the most widely adopted self-hosted automation systems globally, supporting integration with over 1,000 device brands and services using protocols such as MQTT, Z-Wave, Zigbee, and HTTP. Unlike cloud-dependent commercial platforms such as Google Nest, Amazon Alexa, or Apple HomeKit, which process data externally and may introduce privacy risks through cloud transmission, Home Assistant processes all data locally on the host device, ensuring that no personal information is sent to or stored on external servers.

By processing data locally, this architecture directly addresses a key issue identified in smart home adoption research, mainly concerns over data privacy and unauthorized third-party access to sensitive household information. According to Gøthesen et al. (2023), privacy and security issues remain important obstacles to smart home adoption, particularly for users who are concerned about personal data being collected and processed in cloud environments. Home Assistant eliminates this issue by ensuring that both device communication and automation processing remain entirely within the local network.

The system is installed as a operating system (OS) image on compatible hardware and then accessed using a web browser over the local network. The official Home Assistant installation documentation recommends running the system on a Raspberry Pi 4 or 5 with at least 2 GB of RAM, a 32 GB A2-class microSD card, and a wired Ethernet connection for stable and reliable performance.  Following installation, the system operates without a monitor (headless) and is typically accessible at homeassistant.local:8123 within one minute when using a Raspberry Pi 4 or 5. The initial setup process involves setting up credentials, a username, and a geographic location, with the location being particularly important for enabling automations based on time events such as sunrise and sunset.

Home Assistant was selected as the core software platform for this project based on several well-supported and practical reasons. Firstly, its local-first design eliminates relying on cloud services and subscription fees, which makes it well suited for a low-cost, self-funded prototype. Secondly, the Lovelace dashboard system offers a highly customizable drag-and-drop interface, allowing devices, sensors, and automation outputs to be arranged and displayed to suit the specific needs of the project. Thirdly, the platform’s automation engine enables conditional logic using sensor data, time schedules, occupancy, and external triggers, which supports the control and monitoring goals of this thesis. Lastly, Home Assistant appears frequently in recent academic research on Raspberry Pi-based IoT systems, reinforcing its credibility as a suitable platform. In comparison with platforms like OpenHAB and Domoticz, Home Assistant provides a wider range of integrations, stronger community support, and better documentation for beginners.

2.3 Raspberry Pi in IoT

The Raspberry Pi is a single-board computer created by the Raspberry Pi Foundation to support computer science education. Over time, it has become one of the most widely used platforms for IoT prototyping and development in both research and industry. The combination of low cost, compact design, and real computing ability makes it useful for many applications, from simple monitoring and educational projects to full home automation systems that control multiple devices at the same time. The Raspberry Pi 4B offers improved processing power, more USB and display ports, dual-band wireless connectivity, and better overall standards compared to earlier models. This makes it suitable for more demanding applications involving multiple sensors and networked systems.

A key technical advantage of the Raspberry Pi for IoT applications is its 40-pin General Purpose Input/Output (GPIO) interface, which enables direct connection with a wide variety of sensors, actuators, relay modules, and components. This makes it ideal for embedded systems that rely on real-time collection of data from environmental sensors. The platform supports programming languages such as Python, Java, C++, Ruby, and C#, giving developers flexibility across different types of applications. In home automation, Raspberry Pi often acts as a central control hub that manages lighting, heating, ventilation and air-condition systems (HVAC), security locks, and environmental monitoring, while communicating with devices using wireless technologies such as Zigbee, Z-Wave, and Wi-Fi. Research also shows that in many Home Automation Systems; Raspberry Pi boards are used as the main embedded computing unit connected to wired sensors. The time when sensors are inactive can be used to perform extra processing tasks, which improves overall system efficiency in home environments.

Novak et al. (2025) carried out a systematic review and  network analysis of smart home security research using Raspberry Pi, analyzing 174 papers from the Web of Science, IEEE, and Scopus databases. Their analysis confirmed that Raspberry Pi is mainly used in smart home security for intrusion detection, surveillance systems, fire detection, and assistive systems for elderly users, with cost-effectiveness being a key advantage repeatedly highlighted in the literature. The study further highlighted two main concerns in Raspberry Pi-based smart home systems: developing security systems that protect users, and ensuring the systems are secure against cyber threats including attacks, unauthorized access, and privacy violations. Evans (2025) additionally confirms that when Home Assistant is deployed on a Raspberry Pi 4, it operates as a fully functional headless home automation server accessible over the local network, requiring only minimal hardware such as a microSD card and a stable Ethernet connection.

Despite these advantages, limitations exist and must be recognized. Mathe et al. (2024) notes that Raspberry Pi may not operate reliably in harsh industrial environments and does not offer the same durability and long-term reliability as industrial control systems. Challenges such as data security, hardware compatibility, and managing large volumes of sensor data are also identified as technical obstacles that need to be addressed in production systems. For the purposes of this thesis-based smart home prototype, the Raspberry Pi 4B provides a well-established, affordable, and technically suitable platform, supported by a large community and clear integration pathways for Home Assistant and IoT sensor networks.

Section 2.4 — IIoT Trends and Relevance

IIoT market scale and growth

The Industrial Internet of Things is one of the fastest-growing technology sectors globally. According to IMARC Group, the Industrial Internet of Things (IIoT) market reached USD 325.7 billion in 2025 and is expected to increase to USD 944.8 billion by 2034, representing a compound annual growth rate (CAGR) of 12.18%.CBI reports that Europe’s IoT market is expected to grow from €179 billion in 2025 to €258 billion by 2029, at a CAGR of 9.62%. Globally, there are around 20 billion active IoT devices in 2025, projected to exceed 40 billion by 2034, with Europe accounting for approximately one-fifth of these devices. Key factors driving this growth are the expansion of 5G connectivity, developments in edge computing, and the rising use of artificial intelligence within IoT systems. Europe’s strong position in IIoT adoption is supported by its established manufacturing industry, government-led digital transformation strategies, and sustainability requirements under initiatives such as the European Green Deal.

IIoT in smart homes and energy relevance

In the residential sector, IoT-based energy management systems are becoming increasingly important. Bagdadee et al. (2025) demonstrated that an IoT-based hybrid renewable energy system integrating solar panels, wind turbines, and battery storage managed through sensors using Zigbee, Z-Wave, and Wi-Fi protocols achieved 80% system efficiency and reduced energy costs by 61% compared to conventional grid-connected systems. Real-time monitoring through IoT sensors enabled adaptive control of energy flows, directly reducing CO₂ emissions by over 61%. This highlights the practical value of IIoT principles being applied at residential environments, where distributed sensing, real-time feedback, and automated operation work together to optimise resource use. According to Mordor Intelligence (2025), the European smart homes market was valued at USD 23.14 billion in 2025 and is expected to grow to USD 32.12 billion by 2031, driven by energy efficiency regulations, automated load-shifting requirements, and dynamic electricity pricing, all of which require the kind of sensor-actuator-controller architecture that Industrial Internet of Things (IIoT) provides.

Scalability, Poland, and the DIY link

Ezugwu et al. (2025) states that home automation developers rely on hardware from multiple manufacturers, which often leads to systems that are not fully scalable or compatible. Differences in IoT hardware standards can cause communication and connectivity problems between devices. The integration of AI with IoT transforms systems from simple monitoring tools into predictive solutions capable of identifying and preventing potential issues within the home environment. However, these advanced systems often demand high processing power and infrastructure investment. CBI (2025) reports that Poland is one of Europe’s fastest-growing IoT markets, with a projected CAGR of 12.06% through 2029, driven by key sectors such as manufacturing, energy, and logistics. For most households, high-level IIoT systems are too expensive and complex to implement. Platforms like Home Assistant, when deployed on a Raspberry Pi 4, help bridge this gap by offering a low-cost, open-source alternative. The system developed in this thesis applies a layered IIoT architecture (sensor, gateway, and control) at a household scale, making IIoT functionality accessible to ordinary users without significant costs.

### 2.4 IIoT Trends and Relevance

The Industrial Internet of Things represents one of the most significant technological developments of the current decade, transforming how industries monitor, control, and optimise physical processes through interconnected sensor networks and real-time data analytics. According to IMARC Group (2025), the global IIoT market reached USD 325.7 billion in 2025 and is projected to grow to USD 944.8 billion by 2034, at a compound annual growth rate of 12.18%. The primary drivers of this expansion include the proliferation of smart city initiatives, advances in edge computing, and the rapid deployment of 5G connectivity. The Centre for the Promotion of Imports (CBI, 2025) reports that Europe's broader IoT market is expected to grow from €179 billion in 2025 to €258 billion by 2029, with Europe holding the largest regional IIoT share globally due to its robust manufacturing base and strong government investment in digital infrastructure. Germany alone is projected to have 84% of its manufacturers investing approximately EUR 10 billion annually in smart manufacturing technologies by 2025 (IMARC, 2025).

A core principle of IIoT that has direct relevance to residential smart home applications is the integration of distributed sensor networks with real-time data processing and automated actuation. Bagdadee et al. (2025) demonstrated this principle at the household scale, showing that an IoT-based hybrid energy system — coordinating solar panels, wind turbines, and battery storage through Zigbee, Z-Wave, and Wi-Fi communication protocols — achieved system efficiency of 80% and reduced energy costs by 61% compared to conventional grid-connected systems. CO₂ emissions were simultaneously reduced by over 61%. The authors attribute these gains to real-time monitoring and adaptive energy management enabled by IoT sensors and controllers, confirming that the architectural logic of IIoT — distributed sensing, centralised decision-making, and automated response — produces measurable efficiency improvements even at the residential level. The Mordor Intelligence Europe Smart Homes Market Report (2026) contextualises this within market trends, projecting the European smart homes market to grow from USD 23.14 billion in 2025 to USD 32.12 billion by 2031, driven substantially by energy performance mandates, dynamic electricity tariffs, and demand for automated load-shifting capabilities.

Beyond energy management, the convergence of AI and IoT — increasingly referred to as AIoT — introduces predictive and adaptive capabilities that extend smart home functionality beyond reactive automation. Ezugwu et al. (2025) note that intelligent smart home automation systems go beyond mere monitoring and control, possessing the capability to predict and prevent potential incidents within the home environment. Their proposed six-layer architecture — encompassing a smart device and sensor layer, a communication and gateway layer, an intelligent management and decision layer, a cloud computing layer, a control layer, and a security layer — directly mirrors the layered architectures developed for industrial IIoT deployments. The CBI (2025) highlights Poland as one of the fastest-growing IoT markets in Europe, with a projected CAGR of 12.06% through 2029, underscoring the regional relevance of IoT adoption trends to the deployment context of this project. While enterprise-grade IIoT systems typically require significant infrastructure investment, the system developed in this thesis applies these same architectural principles at an accessible scale using a Raspberry Pi 4 and Home Assistant, demonstrating that IIoT design logic can be replicated affordably without sacrificing core functionality.

Section 2.5 — Sensor Technologies: DHT22, PIR, Relay

DHT22 temperature and humidity sensor

Khan et al. (2024) deployed DHT sensors on a Raspberry Pi device to collect indoor temperature and humidity data at hourly intervals as part of a predictive energy optimisation framework. The DHT sensor combines a thermistor for temperature measurement with a capacitative element for humidity detection. The collected data was used to train deep learning models — including LSTM, RNN, and GRU networks — to predict environmental conditions and energy consumption one time-step ahead, enabling proactive rather than reactive control of heating and cooling actuators. Ezugwu et al. (2025) classify DHT22 as an environmental sensor used for measuring humidity and temperature in smart home contexts, noting it as an improved alternative to the DHT11 with greater accuracy. In the system developed for this thesis, two DHT22 sensors are deployed — one indoors and one to monitor ambient conditions — providing the environmental data that drives Home Assistant automations for fan and ventilation control.

PIR motion sensor

Gagote et al. (2026) present a PIR-based automatic lighting system designed to detect human motion and activate LED outputs without a microcontroller, demonstrating that the sensor can directly control a circuit through motion detection alone. The PIR sensor works by detecting changes in infrared radiation emitted by the human body — when a person enters the detection zone, the sensor triggers a response. In their prototype, the system successfully detected motion within its effective indoor range with minimal delay, and showed consistent performance for lighting automation applications. Limitations noted include sensitivity to environmental temperature changes and a restricted detection range. Ezugwu et al. (2025) categorise the HC-SR501 PIR as an environmental sensor used for motion detection and home monitoring, noting its use for safety and security through detection and alarm triggering. In the thesis project, the PIR sensor is connected to the Raspberry Pi GPIO pins, allowing Home Assistant to trigger automations — such as activating a relay-controlled device — whenever occupancy is detected.

Relay module and the sense-decide-actuate loop

Mathe et al. (2024) provide a comprehensive review of Raspberry Pi applications in home automation, identifying relay modules as standard components for controlling household appliances from a low-voltage microcontroller signal. Their review documents relay use across numerous automation studies, where the relay acts as an electrically operated switch — allowing the Raspberry Pi to turn on or off mains-voltage devices safely. Ezugwu et al. (2025) similarly document relay modules as core hardware in smart home automation systems, noting that they are essential for translating digital control signals into physical actuation of appliances such as lights, fans, and heating elements. In the system developed for this thesis, the relay module connects to the Raspberry Pi and is controlled through Home Assistant, while an SG90 servo provides additional physical actuation — for example, triggering a mechanical lock or vent. Together, the DHT22, PIR, relay, and servo form a complete sense-decide-actuate loop: the sensors gather data, Home Assistant processes it and applies automation logic, and the actuators respond accordingly.

Section 2.6 — Comparison of Existing Solutions

Commercial smart home barriers

Li and Yigitcanlar et al. (2021) conducted a systematic literature review identifying the primary barriers to smart home adoption. Their findings show that high upfront costs, limited consumer understanding of the technology, and concerns around privacy and data security are the dominant obstacles preventing wider uptake. They note that while energy efficiency, healthcare monitoring, and security are the main motivations for adoption, the risks — including privacy threats, difficulty integrating technology into domestic life, and energy rebound effects — consistently deter households. Valencia-Arias et al. (2023) similarly found that perceived reliability is the most critical adoption variable across all study populations, with users expressing strong reluctance to allow companies access to personal home data. The Mordor Intelligence Europe Smart Homes report (2026) quantifies one aspect of this barrier directly: comprehensive smart home packages average €3,500–8,000 in hardware costs, with professional installation adding a further €1,200–2,500. Payback periods in lower-tariff markets can exceed nine years, making commercial systems financially inaccessible for most middle-income households.

Home Assistant and Raspberry Pi as an alternative

Novak, Picek, and Andročec (2025) reviewed 174 papers on Raspberry Pi-based smart home security systems and consistently identified cost-effectiveness as a defining advantage of the platform. Their analysis shows that Raspberry Pi supports intrusion detection, surveillance, fire detection, and assistance for elderly users at a fraction of commercial system costs. Home Assistant is identified by Mathe et al. (2024) as a leading open-source platform for home automation, used across numerous studies to manage sensors, actuators, and communication protocols under a unified interface. The Home Assistant project itself (2026) reports integration with over 1,000 device brands and 3,500+ integrations, operates entirely on local infrastructure with no mandatory cloud dependency, and is actively maintained by a global open-source community. Critically, it runs effectively on Raspberry Pi hardware, enabling a full home automation hub for under €100 in hardware costs — compared to thousands for commercial alternatives.

Adaptive gap and positioning of this thesis

Rey-Jouanchicot et al. (2025) highlight that current smart home automation systems — including commercial voice assistants with over 320 million units installed globally — remain largely reactive, responding to explicit commands rather than adapting proactively to user behaviour and environmental context. Their systematic review identifies adaptive decision-making as the most significant open challenge in the field: systems must be able to handle changes in device availability, incorporate natural user feedback, and support multiple occupants. Commercial platforms partly address this through cloud-based machine learning, but doing so requires persistent data upload — reinforcing the privacy concerns identified by Li et al. and Valencia-Arias et al. The system developed in this thesis takes a different approach: by running Home Assistant locally on a Raspberry Pi 4, incorporating DHT22 environmental sensing, PIR-based occupancy detection, and relay-controlled actuation, it delivers a functionally adaptive system that processes all data on-device, at low cost, and without dependency on external cloud infrastructure. This positions the project as a practical response to the twin barriers of cost and privacy that continue to limit commercial smart home adoption in the European market.

REFERENCES

- Park, Y., & Han, J. (2025). Smart home advancements for health care and beyond: Systematic review of two decades of user-centric innovation. Journal of Medical Internet Research, 27, e62793. https://doi.org/10.2196/62793

- Zhao, C. (2025). The evolutionary revolution of smart home systems based on AI+IoT. DEAI '25: Proceedings of the 2nd Guangdong-Hong Kong-Macao Greater Bay Area International Conference on Digital Economy and Artificial Intelligence, 1174–1179. https://doi.org/10.1145/3745238.3745421

- Fortune Business Insights. (2026). Smart home market size, share & industry analysis. https://www.fortunebusinessinsights.com/industry-reports/smart-home-market-101900

- Filho, G. P. R., Villas, L. A., Gonçalves, V. P., Pessin, G., Loureiro, A. A. F., & Ueyama, J. (2019). Energy-efficient smart home systems: Infrastructure and decision-making process. Internet of Things, 5, 153–167. https://doi.org/10.1016/j.iot.2018.12.004

- Alayed, W., Akhtar, A., Hassan, W. U., & Zeeshan, A. (2025). Maximizing energy savings in smart homes through artificial neural network-based artificial intelligence solutions. Clean Energy, 9(2), 140–149. https://doi.org/10.1093/ce/zkae113

- Khan, Q. W., Ahmad, R., Rizwan, A., Khan, A. N., Lee, K., & Kim, D. H. (2024). Optimizing energy efficiency and comfort in smart homes through predictive optimization. Energy Reports, 11, 5619–5637. https://doi.org/10.1016/j.egyr.2024.05.038

- Bagdadee, A. H., Rahman, M. S., Al Mamoon, I., Dewi, D. A., Islam, A. K. M. M., & Zhang, L. (2025). Empowering smart homes by IoT-driven hybrid renewable energy integration for enhanced efficiency. Scientific Reports, 15, 41491. https://doi.org/10.1038/s41598-025-25328-2

- Gøthesen, S., Haddara, M., & Kumar, K. N. (2023). Empowering homes with intelligence: An investigation of smart home technology adoption and usage. Internet of Things, 24, 100944. https://doi.org/10.1016/j.iot.2023.100944

- Li, W., Yigitcanlar, T., Erol, I., & Liu, A. (2021). Motivations, barriers and risks of smart home adoption: From systematic literature review to conceptual framework. Energy Research & Social Science, 80, 102211. https://doi.org/10.1016/j.erss.2021.102211

- Valencia-Arias, A., Cardona-Acevedo, S., Gómez-Molina, S., Gonzalez-Ruiz, J. D., & Valencia, J. (2023). Smart home adoption factors: A systematic literature review and research agenda. PLOS ONE, 18(10), e0292558. https://doi.org/10.1371/journal.pone.0292558

- Rey-Jouanchicot, J., Campo, E., Bouraoui, J., Bottaro, A., & Vigouroux, N. (2025). Adaptation in smart home automation systems: A systematic review of decision-making and interaction. Internet of Things, 31, 101588. https://doi.org/10.1016/j.iot.2025.101588

- IoT Analytics. (2025). State of IoT 2025: Number of connected IoT devices growing 14% to 21.1 billion globally. https://iot-analytics.com/number-connected-iot-devices/

- IMARC Group. (2025). Industrial IoT market report. https://www.imarcgroup.com/industrial-iot-market

- CBI. (2025). Industrial Internet of Things: Market potential. https://www.cbi.eu/market-information/outsourcing-itobpo/industrial-internet-things/market-potential

- Mordor Intelligence. (2025). Europe smart homes market. https://www.mordorintelligence.com/industry-reports/europe-smart-homes-market-industry

- Smart Home Expo. (2026). Smart Home & IoT Expo 2026: 2nd edition. https://www.smarthomeexpo.pl

- Ezugwu, A. E., Taiwo, O., Egwuche, O. S., Abualigah, L., Van Der Merwe, A., Pal, J., Saha, A. K., Alzahrani, A. I., Alblehai, F., Greeff, J., & Olusanya, M. O. (2025). Smart homes of the future. Transactions on Emerging Telecommunications Technologies, 36(1), e70041. https://doi.org/10.1002/ett.70041

- Home Assistant. (2026). Home Assistant: Open source home automation. https://www.home-assistant.io

- Novak, M., Picek, R., & Andročec, D. (2025). Overview of smart home security with Raspberry Pi. Proceedings of ICISS '24, 163–169. https://doi.org/10.1145/3700706.3700733

- Home Assistant. (2026). Installing Home Assistant on Raspberry Pi. https://www.home-assistant.io/installation/raspberrypi

- Evans, P. J. (2025). DIY home automation with Raspberry Pi. Raspberry Pi Foundation. https://www.raspberrypi.com/news/diy-home-automation-with-raspberry-pi/

- APA reference: Mathe, S. E., Kondaveeti, H. K., Vappangi, S., Vanambathina, S. D., & Kumaravelu, N. K. (2024). A comprehensive review on applications of Raspberry Pi. Computer Science Review, 52, 100636. https://doi.org/10.1016/j.cosrev.2024.100636

- Gagote, E. O., Gapol, K. J. D., Murillo, S. K. G., Velasquez, H. U., & Fabro, M. N. (2026). Autolight: Smart motion detection lighting system (passive infrared sensor). International Journal of Research and Scientific Innovation, 13(1), 611–621. https://doi.org/10.51244/IJRSI.2026.13010052