UNIVERSITY OF ECONOMY IN BYDGOSZCZ

BUSANI NCUBE

MULTI-INTERFACE SMART HOME CONTROLLER

CHAPTER 1 | INTRODUCTION

1.1 Background

Home automation has been around for longer than most people think. In 1970, X10 became the first home automation platform, marking an important milestone. It allowed digital data to be transmitted through household electrical wiring using radio signals. This marked the beginning of homes becoming more modern and technology based. As technology continued to grow, the phrase “smart home” was first used in 1984 by the American Home Builders Association. By the late 20th century, technology was advancing rapidly. Home computers and early smart appliances became popular in the 1980s and 1990s, and more people started using them at home. During the late 1990s, there was a big increase in home network development, mainly because high-speed internet became widely available. As smartphones became more popular in the late 2000s, smart homes also grew in popularity since they could now be managed through mobile apps.

As Wi-Fi improved and became widely available, smart home devices became easier to use. With mobile networks such as 4G and 5G, users were able to operate their home devices remotely using mobile applications. IoT home systems connect different home devices, allowing them to work together and be controlled easily, making life more convenient, comfortable, safe, and energy efficient. This growth is reflected in market data - the global smart home market was worth USD 147.52 billion in 2025 and is expected to reach USD 848.47 billion by 2034, growing about 21.4% each year. In 2025, North America held the largest share of the smart home market at 31.7%, while Asia Pacific is projected to experience the fastest growth over the coming years.

Home Automation Systems (HASs), which are part of the smart grid and work with Internet of Things (IoT), are one of the best solutions for improving energy efficiency. In early stages of smart home development, the focus was on automating devices and making life easier for users. Over time, saving energy and reducing costs became more important. Research shows that smart home systems using predictive optimization can save energy costs by about 35.98% to 38.22%. In addition to saving money, smart homes reduce environmental impact. In a simulated smart home environment, the system outperformed conventional energy systems by increasing efficiency by up to 72.3%, lowering energy costs by up to 61%, and reducing CO₂ emissions by more than 61%.

The costs of smart home technology (SHT) have been widely discussed in academic research. Some studies show that cost plays an important role in whether people choose to adopt SHT, and lack of familiarity can make people worried about the price. People face several barriers when adopting smart homes, such as lack of trust, poor understanding of the technology, money concerns, worries about privacy and security, fear of technology, and negative social influence. Technology adoption depends on how people understand and perceive its benefits compared to the risks. Even though there are many advantages, some users still worry about losing control and facing security issues. Concerns around security, privacy, and trust continue to be among the most significant obstacles preventing wider smart home technology adoption

The Internet of Things (IoT) refers to a network of devices connected to the internet that can share data and be programmed to do tasks. It plays a central role in the development of modern smart homes. Smart home systems have recently grown in popularity due to IoT. These systems make life easier and more efficient by using devices such as thermostats and voice assistants to improve comfort and convenience. The number of connected IoT devices worldwide reached 18.5 billion in 2024, showing a 12% rise from 2023. It is expected that IoT devices will continue to grow by 14%, reaching 21.1 billion by the end of 2025 and by 2030, IoT devices are predicted to reach 39 billion, growing at a rate of 13.2% from 2025. Artificial Intelligence is expected to play a major role in this growth because it increases the demand for device data. This growth is especially clear in the voice assistant market, where over 320 million devices are installed in homes worldwide.

This project addresses these challenges by creating a low-cost, multi-interface smart home controller built around a Raspberry Pi 4 running Home Assistant, an ESP32 microcontroller for sensor integration via MQTT, and an ESP32-S3 paired with a Grove Vision AI V2 module for on-device artificial intelligence vision detection. Together with a Waveshare touchscreen for local control, the system demonstrates that a fully functional, AI-enhanced smart home platform can be built affordably without dependence on costly commercial alternatives.

1.2 Significance of the Study

IoT and smart home technologies are growing quickly around the world. The global Industrial IoT market reached USD 325.7 billion in 2025 and is projected to rise to USD 944.8 billion by 2034 with Europe holding the largest share of this market. Europe is one of the most important markets for IoT development because of its strong industry, government support, and increasing use of digital technology in many sectors. At the same time, the smart home market in Europe is expected to grow from USD 23.14 billion in 2025 to more than USD 32 billion by 2031, thanks to energy efficiency laws and rising demand from buyers.

In Poland, the IoT market is growing quickly. It is projected to go from €5.96 billion in 2025 to €9.39 billion in 2029, at a rate of 12.06% per year, driven by strong demand from manufacturing, energy, and transport industries. The use of IoT is growing in Poland as many industries use it to become more efficient and productive. The rising interest in smart homes was clear at the 2026 Smart Home and IoT Expo in Warsaw, which attracted 8,044 professionals from 31 countries—a 47% increase compared to the previous year.

Despite the rising popularity of smart home technology, cost remains a significant barrier to widespread use. Commercial smart home setups in Europe typically cost between EUR 3,500 and EUR 8,000, making them unaffordable for many households in Poland and other Eastern European countries. This project addresses that challenge by developing a low-cost smart home system using a Raspberry Pi and Home Assistant, offering households in Poland a more affordable alternative to high priced commercial systems.

1.3 Aim of the Project

The aim of this project is to design and build a low-cost, multi-interface smart home automation system using a Raspberry Pi 4 running Home Assistant as the central control hub. The system integrates an ESP32 microcontroller for environmental sensing and device actuation, communicating over MQTT, alongside an ESP32-S3 paired with a Grove Vision AI V2 module to provide on-device artificial intelligence vision detection. A Waveshare touchscreen interface offers local control in addition to remote access through a web dashboard, while a relay module enables automated switching of connected electrical devices. Together, these components demonstrate that a fully functional, AI-enhanced smart home system can be built affordably, without dependence on costly commercial alternatives.

1.4 Objectives

The following objectives were set to guide the development of this project:

- To install and configure Home Assistant OS on a Raspberry Pi 4 as the central platform for the smart home system.

- To integrate an ESP32 microcontroller for real-time temperature monitoring, publishing sensor data to Home Assistant via MQTT.

- To connect and configure a PIR motion sensor to trigger automated responses based on occupancy detection.

- To wire and control a relay module for switching electrical devices through the Home Assistant dashboard.

- To integrate an ESP32-S3 with a Grove Vision AI V2 module for on-device person/object detection and automation triggering.

- To configure a Waveshare touchscreen for local dashboard interaction and evaluate the complete system through functional testing and user feedback.

1.5 Scope and Limitations

The focus of this project is to build a functional, multi-interface smart home prototype for academic purposes. The system includes temperature monitoring, motion-based occupancy detection, automated device switching, AI-based vision detection, and both local touchscreen and remote dashboard access. The project does not aim to replace commercial smart home systems but to demonstrate that a comparably capable, AI-enhanced alternative can be built at low cost using accessible hardware.

This project has several limitations. The system depends entirely on a stable Wi-Fi connection; if the network is unavailable, both remote dashboard access and MQTT-based sensor communication are lost. The system operates at prototype level and has not been tested for long-term reliability or deployment in a real home environment. The Grove Vision AI V2 module's detection accuracy is dependent on lighting conditions and camera placement, which were not exhaustively tested across all environments. Security features such as user authentication hardening and encrypted MQTT communication are outside the scope of this project and are recommended for future development.

1.6 Structure of Thesis

This thesis is organised into six chapters. Chapter 1 introduces the project by providing background context, establishing the significance of the study, and defining the aim, objectives, scope, and limitations. Chapter 2 presents a review of existing literature covering smart home systems, the Home Assistant platform, Raspberry Pi in IoT applications, IIoT trends, sensor and AI vision technologies, and a comparison of existing commercial systems. Chapter 3 describes the system design, including the architecture, hardware selection, automation logic, dashboard layout, and wiring diagrams. Chapter 4 describes the implementation of the core hardware system, including sensor integration, MQTT communication, relay control, and dashboard development. Chapter 5 details the implementation of the AI vision component, presents full system integration testing, and shares results from user evaluation. Chapter 6 concludes the thesis by summarising what was achieved, evaluating performance against the objectives, noting limitations, suggesting future improvements, and reflecting on the project's relevance to IIoT trends.

CHAPTER 2 | LITERATURE REVIEW

2.1 Overview of Smart Home Systems

The idea of a smart home has changed a lot over the past few decades, moving from simple wired control systems to advanced connected environments using Internet Of Things (IoT) and artificial intelligence. A smart home can be defined as a connected living space where appliances and systems can be controlled, monitored, and managed remotely through devices like smartphones, tablets, and voice-controlled interfaces. These systems combine technologies such as IoT, cloud computing, artificial intelligence, and embedded sensors to enable centralized management and remote control of household devices. These systems primarily provide better comfort, convenience, energy efficiency, and security, while also automating daily tasks that would otherwise need direct human involvement.

Smart home development began in the 1970s with the X10 platform, a system that used existing electrical wiring to transmit digital signals.The concept of smart homes grew further in the late 1990s with the rise of high-speed internet, and it accelerated again in the 2000s with the spread of smartphones and IoT devices. Research activity has reflected this development trend. According to Park and Han (2025), smart home publications increased sharply after 2014, with most academic work produced between 2018 and 2023, indicating growing technological maturity and broader applications.

The global market also demonstrates this continued growth. The smart home market was valued at USD 147.52 billion in 2025 and is projected to reach USD 848.47 billion by 2034, growing at a compound annual growth rate of 21.40% . North America currently holds the largest market share at 31.70%, while the Asia Pacific region is expected to grow the fastest due to urbanisation, rising incomes, and increased adoption of IoT devices. European adoption of smart home technologies is shaped by factors such as hedonic motivation (the enjoyment from using the technology), price value, and social influence, while energy management is becoming important due to increasing energy costs..

From an infrastructure perspective, Home Automation Systems rely on communication networks to collect, distribute, and process data within the home. Wired systems have several limitations, such as high installation costs, limited coverage range, and a lack of mobility. Wireless Sensor and Actuator Networks (WSANs) are a promising alternative to wired systems, offering better scalability, greater flexibility, and simpler integration with other networks. Research has shown that shifting from wired to wireless networks supports the use of computational intelligence to enhance decision-making in home automation.

Modern smart home systems use a layered architecture that consists of perception, network, edge, and cloud layers, along with an application layer that provides the user interface. The integration of AI and IoT has improved smart home systems by allowing devices to learn user habits and preferences, and automatically adjust to daily routines and environmental conditions without manual control. Ezugwu et al. (2025) notes that, smart home systems are likely to evolve into ambient intelligence environments that automatically respond to occupants’ needs. This shift will be driven by machine learning and deep learning, enabling more personalized and predictive automation.

2.2 Home Assistant as a Platform

Home Assistant is a free, open-source home automation platform developed and maintained by the Open Home Foundation, designed around the principles of local data control and user privacy. It has become one of the most widely adopted self-hosted automation systems globally, supporting integration with over 1,000 device brands and services using protocols such as MQTT, Z-Wave, Zigbee, and HTTP. Unlike cloud-dependent commercial platforms such as Google Nest, Amazon Alexa, or Apple HomeKit, which process data externally and may introduce privacy risks through cloud transmission, Home Assistant processes all data locally on the host device, ensuring that no personal information is sent to or stored on external servers.

By processing data locally, this architecture directly addresses a key issue identified in smart home adoption research, mainly concerns over data privacy and unauthorized third-party access to sensitive household information. According to Gøthesen et al. (2023), privacy and security issues remain important obstacles to smart home adoption, particularly for users who are concerned about personal data being collected and processed in cloud environments. Home Assistant eliminates this issue by ensuring that both device communication and automation processing remain entirely within the local network.

The system is installed as a operating system (OS) image on compatible hardware and then accessed using a web browser over the local network. The official Home Assistant installation documentation recommends running the system on a Raspberry Pi 4 or 5 with at least 2 GB of RAM, a 32 GB A2-class microSD card, and a wired Ethernet connection for stable and reliable performance.  Following installation, the system operates without a monitor (headless) and is typically accessible at homeassistant.local:8123 within one minute when using a Raspberry Pi 4 or 5. The initial setup process involves setting up credentials, a username, and a geographic location, with the location being particularly important for enabling automations based on time events such as sunrise and sunset.

Home Assistant was selected as the core software platform for this project based on several well-supported and practical reasons. Firstly, its local-first design eliminates relying on cloud services and subscription fees, which makes it well suited for a low-cost, self-funded prototype. Secondly, the Lovelace dashboard system offers a highly customizable drag-and-drop interface, allowing devices, sensors, and automation outputs to be arranged and displayed to suit the specific needs of the project. Thirdly, the platform’s automation engine enables conditional logic using sensor data, time schedules, occupancy, and external triggers, which supports the control and monitoring goals of this thesis. Lastly, Home Assistant appears frequently in recent academic research on Raspberry Pi-based IoT systems, reinforcing its credibility as a suitable platform. In comparison with platforms like OpenHAB and Domoticz, Home Assistant provides a wider range of integrations, stronger community support, and better documentation for beginners.

2.3 Raspberry Pi in IoT

The Raspberry Pi is a single-board computer created by the Raspberry Pi Foundation to support computer science education. Over time, it has become one of the most widely used platforms for IoT prototyping and development in both research and industry. The combination of low cost, compact design, and real computing ability makes it useful for many applications, from simple monitoring and educational projects to full home automation systems that control multiple devices at the same time. The Raspberry Pi 4B offers improved processing power, more USB and display ports, dual-band wireless connectivity, and better overall standards compared to earlier models. This makes it suitable for more demanding applications involving multiple sensors and networked systems.

A key technical advantage of the Raspberry Pi for IoT applications is its 40-pin General Purpose Input/Output (GPIO) interface, which enables direct connection with a wide variety of sensors, actuators, relay modules, and components. This makes it ideal for embedded systems that rely on real-time collection of data from environmental sensors. The platform supports programming languages such as Python, Java, C++, Ruby, and C#, giving developers flexibility across different types of applications. In home automation, Raspberry Pi often acts as a central control hub that manages lighting, heating, ventilation and air-condition systems (HVAC), security locks, and environmental monitoring, while communicating with devices using wireless technologies such as Zigbee, Z-Wave, and Wi-Fi. Research also shows that in many Home Automation Systems; Raspberry Pi boards are used as the main embedded computing unit connected to wired sensors. The time when sensors are inactive can be used to perform extra processing tasks, which improves overall system efficiency in home environments.

Novak et al. (2025) carried out a systematic review and  network analysis of smart home security research using Raspberry Pi, analyzing 174 papers from the Web of Science, IEEE, and Scopus databases. Their analysis confirmed that Raspberry Pi is mainly used in smart home security for intrusion detection, surveillance systems, fire detection, and assistive systems for elderly users, with cost-effectiveness being a key advantage repeatedly highlighted in the literature. The study further highlighted two main concerns in Raspberry Pi-based smart home systems: developing security systems that protect users, and ensuring the systems are secure against cyber threats including attacks, unauthorized access, and privacy violations. Evans (2025) additionally confirms that when Home Assistant is deployed on a Raspberry Pi 4, it operates as a fully functional headless home automation server accessible over the local network, requiring only minimal hardware such as a microSD card and a stable Ethernet connection.

Despite these advantages, limitations exist and must be recognized. Mathe et al. (2024) notes that Raspberry Pi may not operate reliably in harsh industrial environments and does not offer the same durability and long-term reliability as industrial control systems. Challenges such as data security, hardware compatibility, and managing large volumes of sensor data are also identified as technical obstacles that need to be addressed in production systems. For the purposes of this thesis-based smart home prototype, the Raspberry Pi 4B provides a well-established, affordable, and technically suitable platform, supported by a large community and clear integration pathways for Home Assistant and IoT sensor networks.

2.4 IIoT Trends and Relevance

The Industrial Internet of Things is one of the fastest-growing technology sectors globally. According to IMARC Group, the Industrial Internet of Things (IIoT) market reached USD 325.7 billion in 2025 and is expected to increase to USD 944.8 billion by 2034, representing a compound annual growth rate (CAGR) of 12.18%. This expansion is driven by the increasing number of smart city initiatives, developments in edge computing, and the widespread deployment of 5G networks. The Centre for the Promotion of Imports (CBI, 2025) reports that Europe's broader IoT market is expected to grow from €179 billion in 2025 to €258 billion by 2029, with Europe holding the largest regional IIoT share globally due to its robust manufacturing base and strong government investment in digital infrastructure.

A core principle of IIoT that has direct relevance to residential smart home applications is the integration of distributed sensor networks with real-time data processing. Bagdadee et al. (2025) validated this concept, showing that an IoT-based hybrid energy system integrating solar panels, wind turbines, and battery storage through Zigbee, Z-Wave, and Wi-Fi communication protocols achieved 80% system efficiency and reduced energy costs by 61% compared to conventional grid-connected systems. CO₂ emissions were also reduced by over 61%. These improvements are credited to real-time monitoring and adaptive energy management using IoT sensors and controllers, showing that IIoT principles such as distributed sensing, centralised control, and automated actuation produce measurable efficiency gains in residential environments. According to Mordor Intelligence (2025), the European smart homes market was valued at USD 23.14 billion in 2025 and is expected to grow to USD 32.12 billion by 2031, driven by energy efficiency regulations, automated load-shifting requirements, and dynamic electricity pricing, all of which require the kind of sensor-actuator-controller architecture that Industrial Internet of Things (IIoT) provides.

In addition to energy management, the integration of AI and IoT enables predictive and adaptive features that enhance smart home systems beyond simple reactive automation. Ezugwu et al. (2025) states that advanced smart home systems extend beyond basic monitoring and control by enabling the prediction and prevention of potential incidents in the home. Their six-layer architecture consisting of smart devices and sensors, communication and gateway, intelligent management and decision-making, cloud computing, control, and security layers—closely aligns with the layered frameworks commonly applied in industrial IIoT systems. CBI (2025) identifies Poland as one of the fastest-growing IoT markets in Europe, with a projected CAGR of 12.06% through 2029, highlighting the regional relevance of IoT adoption trends to the deployment context of this project. While enterprise IIoT systems often demand significant infrastructure investment, this thesis implements the same architectural principles at a lower cost using a Raspberry Pi 4 and Home Assistant, showing that IIoT design concepts can be reproduced affordably while maintaining core functionality.

2.5 Sensor and AI Vision Technologies: ESP32, PIR, Relay, and Grove Vision AI V2

Sensors and edge AI modules form the foundation of modern IoT systems by converting physical environmental conditions into digital data that can be processed, recorded, and used for decision-making. Ezugwu et al. (2025) defines a sensor as a device that detects certain external factors in the environment and responds accordingly, with sensor technology forming the foundation of smart home automation systems. Environmental and occupancy sensors are considered the primary data-gathering layer in IoT systems and are typically installed on surfaces like walls, doors, and furniture to enable continuous monitoring of home environments. In this thesis, sensing and actuation functionality is implemented using an ESP32 microcontroller integrating an internal temperature sensor, a Passive Infrared (PIR) motion sensor, and an electromagnetic relay module, complemented by an ESP32-S3 paired with a Grove Vision AI V2 module for on-device artificial intelligence vision detection.

The ESP32 is a low-cost, dual-core microcontroller with integrated Wi-Fi and Bluetooth connectivity, widely used in IoT prototyping due to its processing capability and large number of GPIO pins. Serepas et al. (2025) note that ESP32-based architectures have become increasingly common in smart home and IoT gateway research due to their integrated Wi-Fi connectivity and ability to handle MQTT-based communication directly on-device at minimal cost and power consumption, eliminating the need for separate networking hardware. In this project, the ESP32's internal temperature sensor and connected PIR module both publish data to Home Assistant over MQTT using autodiscovery, allowing sensor entities to be created automatically without manual YAML configuration.

The Passive Infrared (PIR) sensor detects motion by measuring changes in infrared radiation emitted by warm bodies within its detection range. Gagote et al. (2026) developed a PIR-based automatic lighting system designed to activate an LED circuit upon detection of motion, demonstrating consistent indoor operation with minimal delay. Although limitations such as sensitivity to temperature fluctuations and a limited detection range exist, the study confirms that PIR sensors are viable low-cost, energy-efficient components for occupancy-based automation.

The relay module acts as an interface between the ESP32's low-voltage control signals and higher-voltage household appliances, functioning as an electrically controlled switch that enables digital signals to safely control connected circuits. Mathe et al. (2024) document relay use across numerous Raspberry Pi and microcontroller-based home automation implementations, identifying it as a standard hardware component for appliance control in both research prototypes and practical deployments. Ezugwu et al. (2025) similarly list relay boards as essential elements of the smart home hardware stack, used to translate digital control commands into physical actuation of devices such as lights, fans, and heating elements.

The Grove Vision AI V2, built around the Himax WiseEye2 HX6538 processor, represents a significant advancement over traditional binary sensors by performing on-device machine learning inference for object and person detection without relying on cloud processing. Zhao (2025) highlights those edge AI vision modules are increasingly integrated into smart home systems to enable more context-aware automation, moving beyond simple motion detection toward identifying what is causing the motion. By pairing the Grove Vision AI V2 with an ESP32-S3, this thesis extends the sense-decide-actuate loop beyond binary occupancy detection: the ESP32 and PIR sensor gather baseline environmental and occupancy data, the Vision AI module provides higher-level scene understanding, Home Assistant processes this combined input through user-defined automation logic, and the relay module responds by controlling connected devices accordingly.

2.6 Comparison of Existing Solutions

Commercial smart home barriers

Li et al. (2021) conducted a systematic literature review identifying the primary barriers to smart home adoption. Their findings show that high upfront costs, limited consumer understanding of the technology, and concerns around privacy and data security are the dominant obstacles preventing wider uptake. They note that while energy efficiency, healthcare monitoring, and security are the main motivations for adoption, the risks — including privacy threats, difficulty integrating technology into domestic life, and energy rebound effects — consistently deter households. Valencia-Arias et al. (2023) similarly found that perceived reliability is the most critical adoption variable across all study populations, with users expressing strong reluctance to allow companies access to personal home data. The Mordor Intelligence Europe Smart Homes report (2026) quantifies one aspect of this barrier directly: comprehensive smart home packages average €3,500–8,000 in hardware costs, with professional installation adding a further €1,200–2,500. Payback periods in lower-tariff markets can exceed nine years, making commercial systems financially inaccessible for most middle-income households.

Home Assistant and Raspberry Pi as an alternative

Novak, Picek, and Andročec (2025) reviewed 174 papers on Raspberry Pi-based smart home security systems and consistently identified cost-effectiveness as a defining advantage of the platform. Their analysis shows that Raspberry Pi supports intrusion detection, surveillance, fire detection, and assistance for elderly users at a fraction of commercial system costs. Home Assistant is identified by Mathe et al. (2024) as a leading open-source platform for home automation, used across numerous studies to manage sensors, actuators, and communication protocols under a unified interface. The Home Assistant project itself (2026) reports integration with over 1,000 device brands and 3,500+ integrations, operates entirely on local infrastructure with no mandatory cloud dependency, and is actively maintained by a global open-source community. Critically, it runs effectively on Raspberry Pi hardware, enabling a full home automation hub for under €100 in hardware costs — compared to thousands for commercial alternatives.

Adaptive gap and positioning of this thesis

Rey-Jouanchicot et al. (2025) highlight that current smart home automation systems — including commercial voice assistants with over 320 million units installed globally — remain largely reactive, responding to explicit commands rather than adapting proactively to user behaviour and environmental context. Their systematic review identifies adaptive decision-making as the most significant open challenge in the field: systems must be able to handle changes in device availability, incorporate natural user feedback, and support multiple occupants. Commercial platforms partly address this through cloud-based machine learning, but doing so requires persistent data upload — reinforcing the privacy concerns identified by Li et al. and Valencia-Arias et al. The system developed in this thesis takes a different approach: by running Home Assistant locally on a Raspberry Pi 4, incorporating ESP32-based environmental sensing, PIR-based occupancy detection, relay-controlled actuation, and Grove Vision AI V2-based edge vision detection it delivers a functionally adaptive system that processes all data on-device, at low cost, and without dependency on external cloud infrastructure. This positions the project as a practical response to the twin barriers of cost and privacy that continue to limit commercial smart home adoption in the European market.

CHAPTER 3 – SYSTEM DESIGN

3.1 System Architecture

The smart home system developed in this thesis follows a distributed sensor-actuator architecture centred around a Raspberry Pi 4B running Home Assistant OS as the central control hub. The overall system is illustrated in Figure 1 below.

Figure 1: System Architecture Diagram of the Multi-Interface Smart Home Controller

The architecture consists of four main layers. The sensing and actuation layer is implemented on an ESP32-WROOM-32 microcontroller, which integrates an internal temperature sensor, a Passive Infrared (PIR) motion sensor, a relay module for device switching, a BH1750 ambient light sensor, and an MQ-2 gas and smoke sensor. A second microcontroller, the ESP32-S3, handles the AI vision layer by interfacing with a Grove Vision AI V2 module to perform on-device person and object detection. Both microcontrollers communicate with the central hub wirelessly using the MQTT protocol over Wi-Fi, publishing sensor data and receiving control commands through the Mosquitto broker running on the Raspberry Pi.

The processing and automation layer is handled entirely by Home Assistant OS on the Raspberry Pi 4B. Home Assistant receives all incoming MQTT data, creates sensor entities automatically through MQTT autodiscovery, and executes user-defined automations based on sensor states and thresholds. All data processing occurs locally on the device with no dependency on external cloud services.

The interface layer provides two distinct access methods. A Waveshare 4.3 inch HDMI touchscreen connects directly to the Raspberry Pi, displaying the Home Assistant Lovelace dashboard in kiosk mode for local control. Remote access is provided through any phone or laptop browser via the local network at homeassistant.local:8123, allowing users to monitor sensor data, control the relay, and view automation history from anywhere on the network.

3.2 Hardware Selection and Justification

Each hardware component in this project was selected based on specific technical requirements, cost considerations, and compatibility with the Home Assistant platform and MQTT communication protocol.

Raspberry Pi 4B (4GB RAM) was selected as the central hub because it provides sufficient processing power to run Home Assistant OS continuously while simultaneously managing multiple MQTT connections, executing automations, and serving the Lovelace dashboard to connected devices. Its 4GB RAM configuration ensures stable operation under the combined workload of the Mosquitto broker, Home Assistant core, and the HAOS Kiosk display add-on. As confirmed by Novak et al. (2025) and Mathe et al. (2024), the Raspberry Pi 4B is the most widely used single-board computer in smart home research due to its balance of cost, performance, and community support.

ESP32-WROOM-32 was selected as the primary sensor node because it integrates Wi-Fi connectivity, a dual-core 240MHz processor, and multiple GPIO pins in a single low-cost module. Its built-in temperature sensor provides an immediately available data source without additional hardware, and its GPIO interface supports both digital sensors (PIR, relay) and analog sensors (MQ-2) as well as I2C communication (BH1750). Serepas et al. (2025) confirms that ESP32-based architectures are well-suited for MQTT-based smart home sensor nodes due to their integrated connectivity and low power consumption.

ESP32-S3 was selected for the AI vision subsystem because it offers enhanced processing capabilities compared to the standard ESP32, including support for vector instructions that accelerate machine learning inference tasks. This makes it the appropriate pairing for the Grove Vision AI V2 module, which requires a capable host microcontroller to receive, process, and forward detection results via MQTT.

Grove Vision AI V2 (Himax WiseEye2 HX6538) was selected to provide on-device artificial intelligence vision detection without cloud dependency. The module performs person and object detection inference locally, eliminating latency and privacy concerns associated with cloud-based vision processing. This directly addresses the adaptive decision-making gap identified by Rey-Jouanchicot et al. (2025), moving the system beyond simple binary motion detection toward context-aware scene understanding.

PIR Motion Sensor (HC-SR505) was selected for occupancy detection due to its low cost, simple three-pin interface, and proven reliability in indoor environments. Gagote et al. (2026) confirm PIR sensors as viable, energy-efficient components for occupancy-based automation in residential settings. The sensor connects directly to an ESP32 GPIO pin and requires no additional driver circuitry.

Relay Module (SRD-05VDC-SL-C) was selected to provide physical device switching capability, completing the sense-decide-actuate loop of the smart home system. The module's built-in transistor driver allows direct control from a 3.3V ESP32 GPIO signal while switching a 5V coil, eliminating the need for a level converter or external driver circuit.

BH1750 Light Sensor was selected over a simpler LDR voltage divider because it communicates over I2C and returns calibrated lux values directly, removing the need for manual analog calibration. This produces more meaningful, reproducible data suitable for threshold-based automations and graphing in the Home Assistant dashboard.

MQ-2 Gas and Smoke Sensor was selected to add a safety monitoring dimension to the system. It detects LPG, methane, smoke, and carbon monoxide, enabling immediate push notification automations when gas or smoke concentration crosses a defined threshold. This extends the system's functionality beyond comfort and convenience into home safety monitoring.

Waveshare 4.3 inch HDMI Touchscreen was selected to provide a local physical interface, fulfilling the "multi-interface" requirement of the thesis title. The display connects via HDMI for video output and USB for touch input, and runs the Home Assistant dashboard in kiosk mode through the HAOS Kiosk Display add-on.

3.3 Home Assistant Configuration Plan

Home Assistant serves as the central software platform for this project, running on the Raspberry Pi 4B as a fully self-hosted, local automation hub. The configuration of Home Assistant is organised across four main areas: MQTT integration and sensor entity management, the Lovelace dashboard, automation logic, and the touchscreen kiosk interface.

MQTT Integration and Sensor Entity Management

The Mosquitto MQTT broker is installed as a Home Assistant add-on and configured to accept connections from both ESP32 microcontrollers on the local network. A dedicated MQTT user account with local access credentials is created within Home Assistant to authenticate broker connections securely. Rather than manually defining sensor entities in configuration.yaml files, this project uses MQTT autodiscovery — a feature of Home Assistant that automatically creates sensor entities when a correctly formatted discovery message is published to the homeassistant/ topic prefix. Each ESP32 publishes a JSON discovery payload on startup, containing the entity name, device class, state topic, and unit of measurement. This approach eliminates manual YAML configuration and ensures entities appear in Home Assistant immediately after the microcontroller connects.

Sensor Entities

The following entities are created automatically through MQTT autodiscovery:

- ESP32 Temperature — sensor entity, device class: temperature, unit: °C

- PIR Motion Sensor — binary sensor entity, device class: motion, payload: ON/OFF

- Smart Relay — switch entity, command topic and state topic for bidirectional control

- BH1750 Light Level — sensor entity, device class: illuminance, unit: lx

- MQ-2 Gas Sensor — sensor entity, device class: gas, threshold-based alert

- Grove Vision AI — binary sensor entity, device class: occupancy, payload: ON/OFF

Lovelace Dashboard

A custom Lovelace dashboard named Smart Home Controller is created to display all sensor entities and controls in a single view. The dashboard includes a temperature gauge card, a PIR motion sensor state card, a 24-hour temperature history graph, a light level sensor card, a gas sensor card, a relay switch toggle card, a Vision AI detection state card, a weather forecast card, and a clock card. The dashboard is accessible from any browser on the local network and is also displayed fullscreen on the Waveshare touchscreen through the HAOS Kiosk Display add-on.

Automation Logic

Three core automations are configured in Home Assistant using the visual automation editor:

Automation 1 — Motion and Light Triggered Relay: when the PIR sensor detects motion and the BH1750 light level falls below a defined lux threshold, the relay is switched on. This creates a context-aware lighting automation that only activates when both occupancy and darkness conditions are met simultaneously.

Automation 2 — Temperature Alert: when the ESP32 internal temperature sensor reading exceeds a defined threshold, a push notification is sent to the registered mobile device via the Home Assistant companion app.

Automation 3 — Gas and Smoke Alert: when the MQ-2 sensor reading crosses a defined concentration threshold, an immediate push notification is sent with the message "Possible gas or smoke detected."

Automation 4 — Scheduled Relay Off: the relay is automatically switched off at midnight daily, ensuring no connected device remains on overnight regardless of occupancy state.

Touchscreen Kiosk Configuration

The HAOS Kiosk Display community add-on is installed and configured to launch the Smart Home Controller dashboard fullscreen on the Waveshare 4.3 inch display on system startup. The add-on uses the Luakit browser with OpenBox window manager to render the Home Assistant frontend directly on the Pi's HDMI output. Touch input from the Waveshare display is passed through as a standard USB HID pointer device, enabling tap and scroll interaction with the dashboard without any additional driver configuration.

3.4 Automation Logic Design

Four automations are designed for this system, each following a trigger-condition-action structure within Home Assistant's automation engine.

Automation 1 — Motion and Light Triggered Relay

- Trigger: PIR Motion Sensor state changes to Detected

- Condition: BH1750 light level is below 100 lux

- Action: Switch relay ON

This automation is context-aware — it only activates the relay when someone is present AND the environment is dark enough to require a light. This avoids unnecessary switching during daylight hours.

Automation 2 — Temperature Alert

- Trigger: ESP32 Temperature sensor rises above 55°C

- Condition: None

- Action: Send push notification — "Temperature threshold exceeded"

Automation 3 — Gas and Smoke Alert

- Trigger: MQ-2 sensor reading crosses defined threshold

- Condition: None

- Action: Send immediate push notification — "Possible gas or smoke detected"

Automation 4 — Scheduled Relay Off

- Trigger: Time — 00:00 daily

- Condition: None

- Action: Switch relay OFF

Each automation is configured using the Home Assistant visual automation editor, which provides a no-code interface for defining triggers, conditions, and actions. Screenshots of each automation configuration are included in Chapter 4.

3.5 Dashboard and Touchscreen Interface Design

The Home Assistant Lovelace dashboard was designed to provide a clear, functional overview of all system components in a single view. The dashboard is accessible from two interfaces: remotely via any phone or laptop browser on the local network, and locally through the Waveshare 4.3 inch touchscreen in kiosk mode.

The dashboard layout was built using Home Assistant's custom dashboard editor and includes the following cards arranged in a logical order:

- Temperature Gauge Card — displays the current ESP32 internal temperature reading with a visual gauge, providing an at-a-glance view of the sensor value

- PIR Motion Sensor Card — shows the current motion state (Clear or Detected) as a binary entity card that updates in real time

- 24-Hour Temperature History Graph — displays a line graph of temperature readings over the past 24 hours, allowing the user to observe trends over time

- BH1750 Light Level Card — displays the current ambient light reading in lux

- MQ-2 Gas Sensor Card — displays the current gas concentration reading with a visual indicator

- Relay Switch Card — provides a toggle switch to manually turn the relay on or off directly from the dashboard

- Grove Vision AI Card — displays the current detection state (person detected or clear) from the Vision AI module

- Weather Forecast Card — displays current local weather conditions and a short-term forecast using the Met.no integration

- Clock Card — displays the current time and date

The dashboard was tested from both a laptop browser and the Home Assistant mobile app on a phone, confirming that all cards load correctly and update in real time across both interfaces.

For the touchscreen interface, the HAOS Kiosk Display add-on launches the Smart Home Controller dashboard fullscreen on the Waveshare display at system startup. The touch input is recognised as a standard USB pointer device, enabling tap and scroll interaction. A screenshot of the dashboard on the Waveshare screen and a video demonstration of touch interaction are included in Chapter 4 as implementation evidence.

3.6 Wiring and Connection Diagrams

This section describes the physical wiring connections between all hardware components. Full breadboard photos are included in Chapter 4 as implementation evidence.

ESP32-WROOM-32 Connections

The ESP32-WROOM-32 is mounted on a breadboard and serves as the primary sensor node. All sensor and actuator connections are made using jumper wires on the breadboard as follows:

| Component | ESP32 Pin | Connection Type |
| --- | --- | --- |
| PIR Sensor VCC | VIN (5V) | Power |
| PIR Sensor GND | GND | Ground |
| PIR Sensor OUT | GPIO 14 | Digital Input |
| Relay Module VCC | VIN (5V) | Power |
| Relay Module GND | GND | Ground |
| Relay Module S | GPIO 26 | Digital Output |
| BH1750 VCC | 3.3V | Power |
| BH1750 GND | GND | Ground |
| BH1750 SDA | GPIO 21 | I2C Data |
| BH1750 SCL | GPIO 22 | I2C Clock |
| MQ-2 VCC | VIN (5V) | Power |
| MQ-2 GND | GND | Ground |
| MQ-2 AOUT | GPIO 34 | Analog Input |

ESP32-S3 Connections

The ESP32-S3 connects to the Grove Vision AI V2 module via the Grove connector interface:

| Component | ESP32-S3 Pin | Connection Type |
| --- | --- | --- |
| Grove Vision AI VCC | 3.3V | Power |
| Grove Vision AI GND | GND | Ground |
| Grove Vision AI SDA | GPIO 5 | I2C Data |
| Grove Vision AI SCL | GPIO 6 | I2C Clock |

Raspberry Pi 4B Connections

| Component | Connection | Interface |
| --- | --- | --- |
| Waveshare 4.3" Display | HDMI port | Video output |
| Waveshare Touch Input | USB port | HID pointer |
| Power Supply | USB-C port | 5V 3A |

Power Distribution

The ESP32-WROOM-32 is powered via USB from a laptop or USB power adapter during development. The 5V VIN pin provides power to the PIR sensor, relay module, and MQ-2 sensor. The 3.3V pin provides power to the BH1750 light sensor. The Raspberry Pi 4B is powered separately via its USB-C port with a dedicated 5V 3A power supply.

All wiring connections described above are implemented on a solderless breadboard using male-to-male and male-to-female jumper wires.

CHAPTER 4 | IMPLEMENATION PART 1

4.1 Raspberry Pi Setup and Home Assistant Installation

The Raspberry Pi 4B was set up as the central hub of the smart home system by installing Home Assistant OS on a microSD card and configuring the initial system settings.

The installation began by downloading the official Home Assistant OS image for Raspberry Pi 4 from the Home Assistant website. Raspberry Pi Imager was used to flash the image onto a 32GB microSD card. The Waveshare 4.3 inch HDMI touchscreen was connected to the Pi via HDMI before powering on, and the USB touch cable was connected to one of the Pi's USB ports. The Pi was then powered on using a 5V 3A USB-C power supply.

After approximately two minutes, Home Assistant OS completed its first boot and displayed the system ready message on the Waveshare screen. The system was accessed from a laptop browser by navigating to homeassistant.local:8123, where the initial setup wizard was completed. During setup, a user account was created with a username and password, and the home location was configured for Bydgoszcz, Poland, enabling time-based automations such as sunrise and sunset triggers.

Initial WiFi connectivity was established by connecting the Pi to a mobile hotspot. One challenge encountered during setup was that the Add-ons section did not appear in the Settings menu immediately after installation. This was resolved by navigating directly to the Apps section, which is the updated name for the Add-on store in Home Assistant 2026. A screenshot of the completed Home Assistant dashboard after initial setup is shown in Figure 2.

[Figure 2 — HA dashboard after initial setup]

The Home Assistant version installed was Core 2026.6.0, Supervisor 2026.05.1, and Operating System 17.3, running on a Raspberry Pi 4 (rpi4-64 architecture). These version details were confirmed through the Settings → About page and are shown in Figure 3.

[Figure 3 — Settings → About page showing version numbers]

4.2 Waveshare Touchscreen Integration

The Waveshare 4.3 inch HDMI LCD touchscreen was integrated with the Raspberry Pi to provide a local physical interface for the smart home dashboard.

The display was connected to the Raspberry Pi using an HDMI cable for video output and a USB cable for touch input. On powering up the Pi, the display immediately showed the Home Assistant OS system ready message, confirming that the HDMI output was detected automatically without any additional display configuration. Touch input detection was verified by running the following command in the Home Assistant Terminal add-on:

cat /proc/bus/input/devices

The output confirmed the touchscreen was detected as an input device with the name "wch Touch", registered as mouse0 and event0. This confirmed that the USB touch input was recognised by the operating system without requiring manual driver installation.

To display the Home Assistant dashboard fullscreen on the Waveshare screen, the HAOS Kiosk Display community add-on was installed. Since this add-on is not available in the official add-on store, it was added by navigating to Settings → Apps → three-dot menu → Repositories, and entering the repository URL: https://github.com/puterboy/HAOS-kiosk. After refreshing the store, the HAOS Kiosk Display add-on appeared and was installed.

Before starting the add-on, the Home Assistant username and password were entered in the Configuration tab as required. The add-on was then started, and the Waveshare screen displayed the Smart Home Controller dashboard fullscreen within approximately 30 seconds. Touch interaction was tested by scrolling and tapping cards on the screen, confirming that both display output and touch input were functioning correctly. A photo of the Waveshare screen showing the dashboard is provided in Figure 4, and a short video demonstration of touch interaction was recorded as additional evidence

.

[Figure 4 — Waveshare screen showing HA dashboard in kiosk mode]

One practical consideration noted during setup is that running the kiosk add-on simultaneously with normal browser access occasionally caused Home Assistant to become temporarily unreachable from the laptop browser. This was resolved by setting the kiosk add-on to manual start only, launching it specifically when touchscreen demonstration is required rather than running it continuously.

4.3 ESP32 Internal Temperature Sensor

The ESP32-WROOM-32 microcontroller was configured to read its internal temperature sensor and publish readings to Home Assistant via MQTT, creating the first live sensor entity in the system.

Before flashing any sketch, the CP210x USB-to-UART driver was installed to enable communication between the laptop and the ESP32 on COM4. The ESP32 board support package was installed in Arduino IDE 2 by adding the Espressif board manager URL and installing the esp32 by Espressif Systems package. The PubSubClient library by Nick O'Leary was installed via the Arduino Library Manager to handle MQTT communication.

The temperature sketch was written to perform three functions: connect to the WiFi network, connect to the Mosquitto MQTT broker on the Raspberry Pi, and publish temperature readings every 10 seconds. The ESP32 internal temperature sensor is accessed through the hardware API function temprature_sens_read(), which returns a value in Fahrenheit that is then converted to Celsius using the standard conversion formula. The full sketch is stored in the project GitHub repository under src/esp32_temp_mqtt/.

MQTT autodiscovery was implemented by publishing a JSON configuration payload to the topic homeassistant/sensor/esp32temp/config on broker connection. This payload included the entity name, state topic, device class, and unit of measurement, allowing Home Assistant to create the ESP32 Temperature sensor entity automatically without any manual configuration.

After uploading the sketch, the Serial Monitor confirmed successful WiFi connection, MQTT broker connection, and temperature publishing at 53.33°C as shown in Figure 5. The ESP32 Temperature entity appeared automatically in Home Assistant under Settings → Devices and Services → MQTT within 30 seconds of the sketch running.

[Figure 5a — Serial Monitor showing WiFi connected, MQTT connected, temperature published]

[Figure 5b — Home Assistant showing ESP32 Temperature entity]

The temperature card was added to the Smart Home Controller dashboard as a gauge card, displaying the live reading with a visual scale. A screenshot of the dashboard showing the temperature gauge is provided in Figure 6.

[Figure 6— Dashboard showing temperature gauge card]

4.4 PIR Sensor Integration

The HC-SR505 Passive Infrared motion sensor was integrated with the ESP32-WROOM-32 to provide occupancy detection for the smart home system.

Wiring

The PIR sensor was wired to the ESP32 on a solderless breadboard using three jumper wires. The sensor's positive power pin (+) was connected to the ESP32 VIN pin to supply 5V, the ground pin (-) was connected to ESP32 GND, and the signal output pin (OUT) was connected to GPIO 14. Initial testing with GPIO 13 produced inconsistent readings, which led to switching to GPIO 14 where stable digital readings were achieved. A photo of the wiring setup is shown in Figure 7.

[Figure 7 — breadboard wiring of PIR sensor to ESP32]

Sketch

The PIR sensor sketch was written to read the digital state of GPIO 14 and publish the motion state to Home Assistant via MQTT. MQTT autodiscovery was used to automatically create a binary sensor entity in Home Assistant with device class set to motion, payload ON for detected and OFF for clear. The sensor includes a 60-second warmup delay in the setup function, which was identified as necessary during testing — without this delay, the sensor produced false motion readings immediately after powering on. The full sketch is stored in the repository under src/pir_mqtt/.

One additional challenge encountered during integration was that the HC-SR505 sensor initially appeared faulty, producing no output signal regardless of wiring configuration. Systematic debugging confirmed the code and GPIO wiring were correct by manually applying 5V to GPIO 14 and observing "Motion detected" in the Serial Monitor. The root cause was identified as insufficient warmup time rather than a hardware fault. Once the 60-second warmup delay was added to the sketch, the sensor operated reliably.

Testing

After uploading the sketch and waiting for the warmup period, motion detection was tested by waving a hand in front of the sensor dome. The Serial Monitor confirmed "Motion detected — published ON" messages, and the PIR Motion Sensor entity in Home Assistant updated from Clear to Detected in real time as shown in Figure 8.

Figure 8a: "PIR Motion Sensor entity in Clear state before motion was detected."

Figure 8b: "PIR Motion Sensor entity updated to Detected state in real time after motion was sensed."

The HC-SR505 sensor holds its output HIGH for approximately 3-8 seconds after detecting motion before returning to LOW. This hold time is hardwired into the sensor and was documented as expected behaviour rather than a fault.

4.5 Relay Module Integration

The relay module was integrated with the ESP32-WROOM-32 to provide physical device switching capability, completing the sense-decide-actuate loop of the smart home system.

Wiring

The standard 5V relay module with SRD-05VDC-SL-C relay was wired to the ESP32 using three jumper wires. The module's VCC pin was connected to ESP32 VIN (5V), GND to ESP32 GND, and the signal input pin (S) to GPIO 26. The module's built-in transistor driver accepts the 3.3V logic signal from the ESP32 GPIO pin to switch the 5V relay coil, eliminating the need for a level converter. A photo of the relay wiring is shown in Figure 9.

[INSERT PHOTO — breadboard wiring of relay module to ESP32]

Sketch

The relay was controlled via MQTT by subscribing to the topic home/relay/switch. When Home Assistant publishes "ON" to this topic, the ESP32 sets GPIO 26 HIGH, energising the relay coil and closing the NO (Normally Open) contact. When "OFF" is published, GPIO 26 is set LOW and the relay opens. The relay state is also published back to home/relay/state so Home Assistant can confirm the current state. MQTT autodiscovery was used to create a switch entity in Home Assistant with bidirectional control. The full sketch is stored under src/relay_mqtt/.

Testing

The relay was tested by toggling the switch entity on the Home Assistant dashboard. An audible click was heard from the relay each time it switched, confirming physical actuation. The switch card on the dashboard updated in real time to reflect the current relay state as shown in Figure 10.

[INSERT SCREENSHOT — HA dashboard showing relay switch card toggled ON]

4.6 MQTT Communication Setup

MQTT was used as the primary communication protocol between the ESP32 microcontrollers and Home Assistant, providing a lightweight, reliable messaging layer for all sensor data and control commands.

Mosquitto Broker Installation

The Mosquitto MQTT broker was installed as a Home Assistant add-on through the Apps section. After installation, Start on boot and Watchdog options were enabled to ensure the broker restarts automatically after any system interruption. A dedicated MQTT user account named mqtt_user was created in Home Assistant under Settings → People → Users with local access only permissions. The MQTT integration was then configured in Home Assistant under Settings → Devices and Services, where the Mosquitto broker was automatically discovered and linked with a single confirmation click. A screenshot of the Mosquitto broker running is shown in Figure 11.

[Figure 11 — Mosquitto broker add-on showing Running status]

MQTT Explorer Verification

MQTT Explorer was installed on the development laptop to verify broker connectivity independently of Home Assistant. A connection was established to homeassistant.local on port 1883 using the mqtt_user credentials. On successful connection, the $SYS topic tree appeared showing 56 topics and 91 system messages, confirming the broker was live and receiving data as shown in Figure 12.

[Figure 12 — MQTT Explorer connected showing $SYS topics]

Topic Structure

The following MQTT topics are used in this project:

| Topic | Direction | Description |
| --- | --- | --- |
| home/esp32/temperature | ESP32 → HA | Temperature readings in °C |
| home/esp32/pir | ESP32 → HA | Motion state ON/OFF |
| home/relay/switch | HA → ESP32 | Relay control command ON/OFF |
| home/relay/state | ESP32 → HA | Relay state confirmation |
| home/esp32/light | ESP32 → HA | Light level in lux |
| home/esp32/gas | ESP32 → HA | Gas concentration reading |
| homeassistant/# | ESP32 → HA | MQTT autodiscovery payloads |

Data Flow

Each ESP32 connects to the Mosquitto broker on startup using the mqtt_user credentials. Sensor readings are published to their respective state topics every 10 seconds. Home Assistant subscribes to all sensor topics and updates entity states in real time. Control commands for the relay are published by Home Assistant to the command topic, and the ESP32 confirms the action by publishing the new state back. All communication occurs entirely on the local network with no external internet dependency.

REFERENCES

- Park, Y., & Han, J. (2025). Smart home advancements for health care and beyond: Systematic review of two decades of user-centric innovation. Journal of Medical Internet Research, 27, e62793. https://doi.org/10.2196/62793

- Zhao, C. (2025). The evolutionary revolution of smart home systems based on AI+IoT. DEAI '25: Proceedings of the 2nd Guangdong-Hong Kong-Macao Greater Bay Area International Conference on Digital Economy and Artificial Intelligence, 1174–1179. https://doi.org/10.1145/3745238.3745421

- Fortune Business Insights. (2026). Smart home market size, share & industry analysis. https://www.fortunebusinessinsights.com/industry-reports/smart-home-market-101900

- Filho, G. P. R., Villas, L. A., Gonçalves, V. P., Pessin, G., Loureiro, A. A. F., & Ueyama, J. (2019). Energy-efficient smart home systems: Infrastructure and decision-making process. Internet of Things, 5, 153–167. https://doi.org/10.1016/j.iot.2018.12.004

- Alayed, W., Akhtar, A., Hassan, W. U., & Zeeshan, A. (2025). Maximizing energy savings in smart homes through artificial neural network-based artificial intelligence solutions. Clean Energy, 9(2), 140–149. https://doi.org/10.1093/ce/zkae113

- Khan, Q. W., Ahmad, R., Rizwan, A., Khan, A. N., Lee, K., & Kim, D. H. (2024). Optimizing energy efficiency and comfort in smart homes through predictive optimization. Energy Reports, 11, 5619–5637. https://doi.org/10.1016/j.egyr.2024.05.038

- Bagdadee, A. H., Rahman, M. S., Al Mamoon, I., Dewi, D. A., Islam, A. K. M. M., & Zhang, L. (2025). Empowering smart homes by IoT-driven hybrid renewable energy integration for enhanced efficiency. Scientific Reports, 15, 41491. https://doi.org/10.1038/s41598-025-25328-2

- Gøthesen, S., Haddara, M., & Kumar, K. N. (2023). Empowering homes with intelligence: An investigation of smart home technology adoption and usage. Internet of Things, 24, 100944. https://doi.org/10.1016/j.iot.2023.100944

- Li, W., Yigitcanlar, T., Erol, I., & Liu, A. (2021). Motivations, barriers and risks of smart home adoption: From systematic literature review to conceptual framework. Energy Research & Social Science, 80, 102211. https://doi.org/10.1016/j.erss.2021.102211

- Valencia-Arias, A., Cardona-Acevedo, S., Gómez-Molina, S., Gonzalez-Ruiz, J. D., & Valencia, J. (2023). Smart home adoption factors: A systematic literature review and research agenda. PLOS ONE, 18(10), e0292558. https://doi.org/10.1371/journal.pone.0292558

- Rey-Jouanchicot, J., Campo, E., Bouraoui, J., Bottaro, A., & Vigouroux, N. (2025). Adaptation in smart home automation systems: A systematic review of decision-making and interaction. Internet of Things, 31, 101588. https://doi.org/10.1016/j.iot.2025.101588

- IoT Analytics. (2025). State of IoT 2025: Number of connected IoT devices growing 14% to 21.1 billion globally. https://iot-analytics.com/number-connected-iot-devices/

- IMARC Group. (2025). Industrial IoT market report. https://www.imarcgroup.com/industrial-iot-market

- CBI. (2025). Industrial Internet of Things: Market potential. https://www.cbi.eu/market-information/outsourcing-itobpo/industrial-internet-things/market-potential

- Mordor Intelligence. (2025). Europe smart homes market. https://www.mordorintelligence.com/industry-reports/europe-smart-homes-market-industry

- Smart Home Expo. (2026). Smart Home & IoT Expo 2026: 2nd edition. https://www.smarthomeexpo.pl

- Ezugwu, A. E., Taiwo, O., Egwuche, O. S., Abualigah, L., Van Der Merwe, A., Pal, J., Saha, A. K., Alzahrani, A. I., Alblehai, F., Greeff, J., & Olusanya, M. O. (2025). Smart homes of the future. Transactions on Emerging Telecommunications Technologies, 36(1), e70041. https://doi.org/10.1002/ett.70041

- Home Assistant. (2026). Home Assistant: Open source home automation. https://www.home-assistant.io

- Novak, M., Picek, R., & Andročec, D. (2025). Overview of smart home security with Raspberry Pi. Proceedings of ICISS '24, 163–169. https://doi.org/10.1145/3700706.3700733

- Home Assistant. (2026). Installing Home Assistant on Raspberry Pi. https://www.home-assistant.io/installation/raspberrypi

- Evans, P. J. (2025). DIY home automation with Raspberry Pi. Raspberry Pi Foundation. https://www.raspberrypi.com/news/diy-home-automation-with-raspberry-pi/

- Mathe, S. E., Kondaveeti, H. K., Vappangi, S., Vanambathina, S. D., & Kumaravelu, N. K. (2024). A comprehensive review on applications of Raspberry Pi. Computer Science Review, 52, 100636. https://doi.org/10.1016/j.cosrev.2024.100636

- Gagote, E. O., Gapol, K. J. D., Murillo, S. K. G., Velasquez, H. U., & Fabro, M. N. (2026). Autolight: Smart motion detection lighting system (passive infrared sensor). International Journal of Research and Scientific Innovation, 13(1), 611–621. https://doi.org/10.51244/IJRSI.2026.13010052

- Serepas, F., Papias, I., Christakis, K., Dimitropoulos, N., & Marinakis, V. (2025). Lightweight embedded IoT gateway for smart homes based on an ESP32 microcontroller. Computers, 14(9), 391. https://doi.org/10.3390/computers14090391